Question: Define OSI layers base on the following concept

1. networking devises

2. Protocols

3. Attacks

4. port numbers

Answer:

The OSI (Open Systems Interconnection) model is a 7-layer conceptual model used to understand how data travels between devices over a network. Each layer has specific responsibilities, networking devices, protocols, and possible security threats.

| Layer | OSI Layer | Main Function | Networking Devices | Common Protocols / Technologies | Common Attacks | Port Numbers |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | Application | Provides network services to applications | Proxy, Application Gateway | HTTP, HTTPS, DNS, SMTP, FTP, SSH | Phishing, HTTP injection, application attacks | 80, 443, 53, 25, 21, 22 |
| 6 | Presentation | Data formatting, encryption, compression | Gateway | TLS/SSL, JPEG, PNG, ASCII, Unicode | TLS downgrade, encryption-related attacks | No direct port |
| 5 | Session | Establishes, manages and terminates sessions | Gateway | NetBIOS, RPC, SIP | Session hijacking, session fixation | No direct port |
| 4 | Transport | End-to-end delivery, reliability and segmentation | Firewall, L4 Load Balancer | TCP, UDP | TCP SYN flood, UDP flood | TCP/UDP ports |
| 3 | Network | Logical addressing and routing | Router, Layer-3 Switch | IPv4, IPv6, ICMP, IP | IP spoofing, route attacks | No direct port |
| 2 | Data Link | Frames, MAC addressing and local delivery | Switch, Bridge, NIC | Ethernet, ARP, VLAN, Wi-Fi (802.11) | ARP spoofing, MAC flooding, VLAN attacks | No direct port |
| 1 | Physical | Transmission of raw bits through physical media | Hub, Repeater, Cables | Ethernet physical standards, fiber, radio | Cable tapping, jamming, physical damage | No port |

Important: Port numbers are primarily associated with the Transport Layer (Layer 4). Application-layer services such as HTTP, HTTPS, DNS and SSH use TCP/UDP ports, but the port number itself belongs to the transport-layer communication.

1. Physical Layer — Layer 1

The Physical Layer is responsible for transmitting raw bits (0s and 1s) through physical media such as copper cables, fiber-optic cables, or radio signals.

- Devices: Hub, repeater, cables, connectors

- Technologies: Ethernet physical standards, fiber, radio

- Attack: Cable tapping, signal jamming, physical damage

- Port: None

Example: An Ethernet cable carrying electrical signals between a PC and switch operates at the Physical Layer.

2. Data Link Layer — Layer 2

The Data Link Layer provides communication between devices on the same local network using frames and MAC addresses.

- Devices: Switch, bridge, Network Interface Card (NIC)

- Protocols/Technologies: Ethernet, ARP, VLAN, Wi-Fi

- Attacks: ARP spoofing, MAC flooding, VLAN attacks

- Port: None

Example: A switch uses MAC addresses to determine where to forward an Ethernet frame.

3. Network Layer — Layer 3

The Network Layer handles logical addressing and routing of packets between different networks.

- Devices: Router, Layer-3 switch

- Protocols: IPv4, IPv6, ICMP

- Attack: IP spoofing, route manipulation

- Port: None

Example: A router uses IP addresses to determine where a packet should be forwarded.

4. Transport Layer — Layer 4

The Transport Layer provides end-to-end communication between applications. It uses TCP or UDP and identifies applications using port numbers.

- Devices: Firewall, Layer-4 load balancer

- Protocols: TCP, UDP

- Attacks: TCP SYN flood, UDP flood

- Port numbers: 22, 23, 25, 53, 80, 443, 3389, etc.

For example:

- TCP 22 → SSH

- TCP 23 → Telnet

- TCP 25 → SMTP

- UDP/TCP 53 → DNS

- TCP 80 → HTTP

- TCP 443 → HTTPS

- TCP 3389 → RDP

Important: A port number is not a physical port. It is a logical identifier used by TCP or UDP to identify a service/application endpoint.

5. Session Layer — Layer 5

The Session Layer establishes, manages, and terminates communication sessions between applications.

- Devices: Gateway, session-aware network devices

- Technologies/Protocols: NetBIOS, RPC, SIP

- Attack: Session hijacking, session fixation

- Port: No dedicated port numbers

Example: When a communication session is established between two applications, the Session Layer helps manage that session.

6. Presentation Layer — Layer 6

The Presentation Layer is responsible for data representation, translation, encryption/decryption, and compression.

- Devices: Gateway

- Technologies: TLS/SSL, JPEG, PNG, ASCII, Unicode

- Attacks: TLS downgrade and attacks targeting weak encryption/configuration

- Port: No direct port number

Example: Encryption can transform readable data into an encrypted form before it is transmitted.

Note: In real TCP/IP networks, TLS is commonly associated with application protocols such as HTTPS, rather than being implemented as a standalone OSI Layer 6 protocol.

7. Application Layer — Layer 7

The Application Layer provides network services directly used by applications.

- Devices: Proxy server, application gateway

- Protocols: HTTP, HTTPS, DNS, SMTP, FTP, SSH

- Attacks: Phishing, HTTP injection, application-layer attacks

- Common service ports: 80, 443, 53, 25, 21, 22

Example: When you open a website using a browser, HTTP/HTTPS is used to communicate with the web server.

Easy way to remember

OSI Layers from Layer 7 → Layer 1:

Application → Presentation → Session → Transport → Network → Data Link → Physical

Or:

A P S T N D P

Security attack mapping

This is especially important for your Network Security syllabus:

Layer 7 – Application    	→ HTTP Injection

Layer 6 – Presentation   → TLS/Encryption attacks

Layer 5 – Session        	→ Session Hijacking

Layer 4 – Transport      	→ TCP SYN Flood

Layer 3 – Network        	→ IP Spoofing

Layer 2 – Data Link      	→ ARP Spoofing

Layer 1 – Physical       	→ Cable Tapping / Jamming

Key point:
Port numbers belong to TCP/UDP at the Transport Layer (Layer 4), while services such as HTTP, HTTPS, DNS, and SSH operate at the Application Layer and are commonly associated with specific port numbers.

Question: Define the key differences between IPv4 and IPv6.

Answer:

IPv4 (Internet Protocol version 4) and IPv6 (Internet Protocol version 6) are network-layer protocols used to provide logical addressing and route packets across networks. The main difference is that IPv4 uses 32-bit addresses, while IPv6 uses 128-bit addresses.

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32-bit | 128-bit |
| Address format | Decimal, separated by dots | Hexadecimal, separated by colons |
| Example | 192.168.1.10 | 2001:db8::1 |
| Number of addresses | Approximately 4.3 billion | Approximately 3.4 × 10³⁸ |
| Address notation | Four decimal octets | Eight hexadecimal groups |
| Header | Variable length (20–60 bytes) | Fixed 40 bytes |
| Broadcast | Supports broadcast | No broadcast; uses multicast/anycast |
| Address configuration | Manual or DHCP | SLAAC, DHCPv6, or manual |
| NAT | Commonly used | Generally not required for address conservation |
| Fragmentation | Can be performed by sender and routers | Performed only by the sending host |
| Security | IPsec support available | IPsec support is part of the IPv6 specification, though not mandatory for every deployment |
| Address resolution | ARP | Neighbor Discovery Protocol (NDP) |
| Private/local addressing | Private addresses such as 10.0.0.0/8 | Uses Unique Local Addresses (ULA), such as fc00::/7 |

Key differences explained

1. Address size:
IPv4 uses a 32-bit address, whereas IPv6 uses a 128-bit address. Therefore, IPv6 provides a vastly larger address space.

2. Address format:
IPv4 uses decimal notation:

192.168.1.10

IPv6 uses hexadecimal notation:

2001:db8::1

3. Address availability:
IPv4 has a limited address space, which contributed to widespread use of NAT. IPv6 provides an extremely large address space, reducing the need for NAT for address conservation.

4. Broadcast:
IPv4 supports broadcast communication. IPv6 does not use broadcast; it uses multicast and anycast mechanisms instead.

5. Address configuration:
IPv4 commonly uses manual configuration or DHCP. IPv6 can automatically configure addresses using SLAAC and can also use DHCPv6 or manual configuration.

6. Address resolution:
IPv4 uses ARP (Address Resolution Protocol) to map an IPv4 address to a MAC address on a local network. IPv6 uses Neighbor Discovery Protocol (NDP), which operates through ICMPv6.

In short

IPv4 = 32-bit + decimal + limited address space
IPv6 = 128-bit + hexadecimal + very large address space

point: IPv6 is not simply “IPv4 with longer addresses”; it also changes mechanisms such as neighbor discovery, addressing/configuration, fragmentation, and broadcast handling.

Question: Define MAC Address.

Answer:

A MAC (Media Access Control) address is a unique hardware address assigned to a network interface such as a Network Interface Card (NIC). It is used at the Data Link Layer (Layer 2) of the OSI model to identify a device/interface within a local network.

Key Points

- Full form: Media Access Control address

- OSI Layer: Layer 2 — Data Link Layer

- Purpose: Identifies a network interface on a local network.

- Address size: 48 bits (6 bytes) for the common Ethernet MAC address.

- Format: Usually written as six groups of hexadecimal digits.

- Example: 00:1A:2B:3C:4D:5E

- Used by: Ethernet and other IEEE 802 network technologies.

- Switches: Use MAC addresses to determine where Ethernet frames should be forwarded.

Structure

A typical MAC address contains 6 hexadecimal bytes:

00  : 1A  : 2B  :  3C  :  4D  :  5E

└─────┘ └────────────┘

OUI          Device-specific

The first part identifies the organization/manufacturer (OUI), while the remaining part identifies the network interface.

Example

Suppose a computer has:

IP Address  	→ 192.168.1.10

MAC Address 	→ 00:1A:2B:3C:4D:5E

The IP address is used for logical addressing and routing between networks, while the MAC address is used for delivery within the local network.

In short:

MAC address = Layer 2 hardware/interface address used to identify a network interface on a local network.

Question: Why are open ports considered an attack surface?

Answer:

An open port is considered part of a system's attack surface because it allows network communication with a service running on that port. If the service has a vulnerability, weak configuration, or improper authentication, an attacker may attempt to exploit it.

How it works

Internet / Network

↓

Open Port

↓

Running Service

↓

Potential Vulnerability

↓

Possible Attack

Example

Suppose a server has:

Port 22   → SSH

Port 80   → HTTP

Port 443  → HTTPS

Each open port provides a potential entry point to the corresponding service.

For example, if SSH (port 22) is exposed to the network and has a weak configuration or vulnerable software, an attacker may attempt to compromise that service.

Why more open ports can increase risk

- Each open port may expose a network service.

- Each service may contain vulnerabilities.

- Attackers can scan systems to discover open ports.

- Unnecessary services increase the number of possible entry points.

- Poorly configured services can expose sensitive functionality.

Important point

An open port does NOT automatically mean the system is vulnerable.

An open port simply means that a service is reachable. The actual risk depends on factors such as:

Open port → Service → Version/configuration → Vulnerability → Exploitability

Example

Port 80

↓

HTTP Web Server

↓

Outdated / vulnerable software

↓

Potential security weakness

↓

Attack surface

In short:

Open ports contribute to the attack surface because they expose network-accessible services that attackers can discover, investigate, and potentially exploit.

Question: How to identify open ports? Using which tool

Answer:

Open ports can be identified by using port-scanning tools that check whether a TCP or UDP port on a host is accepting network communication. tool such as Nmap (Network Mapper).

1. Using Nmap

Nmap (Network Mapper) is a commonly used network-scanning tool.

For a system you own or are authorized to test:	nmap 192.168.1.10

Nmap may show something like:

PORT     STATE    SERVICE

22/tcp   open     ssh

80/tcp   open     http

443/tcp  open     https

Here:

- 22/tcp 	 → open → SSH service is reachable.

- 80/tcp 	 → open → HTTP service is reachable.

- 443/tcp → open → HTTPS service is reachable.

2. Understanding the result

Port → State   → Service

22   → open   → SSH

80   → open   → HTTP

443  → open   → HTTPS

If a port is shown as open, it generally means a service is listening and accepting connections on that port.

3. Other ways

Open ports can also be identified using:

- Operating-system commands to view listening services.

- Firewall/security tools to inspect allowed connections.

- Network scanners such as Nmap.

In short

Open ports are identified by scanning a host and checking which TCP/UDP ports are accepting connections. Nmap is one of the most widely used tools for this purpose.

Security note: Only scan systems you own or have explicit permission to test.